JF-TGK01 Timer switch

I.  Product introduction

JF-TGK series micro computer timer switch, they have high integrated circuit, as a latest electronic switch for control of power source program to open and close, can set one circuit with 8 group times to open and close control (two circuits are available for open and close power source, or by turn, and two circuits have 16 program groups to open or close.), can set the control program to open and close in a week as well. The machine has memory function, which can keep time indication and original program setting for over 6 months at the case that it leaves power source. They are widely used for the auto control of road lamp, billboard, lamp box, broadcast and TV equipment, garden-spray device, breeding field illumination, traveling spot illumination, home appliances, and any electrical equipment needs time control. It is a safe and secure electronic switch which is latest, intelligent and can use weak power to control strong power. For example, if it is matched with AC contactor, it can control three-phrase big power electrical equipment.

II、Technical index:

Executive standard: GB/T1436.1-1998; GB/T1453.8-1998

Rated voltage: 220V-50Hz

Rated current: 20A (resistant)

Working temperature: -10℃～55℃

Time error:<0.5 second/day

III. System function introduction

A. Timer switch has 7 keys: week, hour, min, open lock, clock, set, mode;

B. LCD reading and size as picture 1;

C. At clock situation, press mode key can set wanted working mode, the setting order as below:

(Note: First open lock, and keep pressing it for 4 times.)

OFF-Auto-ON (close-auto-open)

OFF-: Output of power supply, at close situation.

Auto-: Input of power supply, at close situation.

Iv. Operation methods

A. Auto lock without operation after the machine open for 15s.

Only keep pressing open lock key for 4 times to make “a” disappear, then all the keys are available, and when “a” appears after lock at the down left corner of LCD, all the operations of keys are invalid.,

B. when setting time, if you want to cancel the preset time, you can press open lock key to delete, and the time shows “——:——”. Press open lock key again, then return the original time.

C. Adjust week and time

Keep pressing open lock key for 4times to make a disappear, and press week key to adjust, and then press the hour and minute key to adjust the standard Beijing time.

D、Set week and time

Set week: first open lock, then press set key, and LCD shows “1ON”,and the time shows “——:——”，and press week key, the first values shall all open, showing: MO, TU, WE, TH, Fr, SA, SU.

Press week key again, and the number value to be “MO”,

Press again to be “TU”, till appears “SU”.

Press again to be MO, TU, WE, TH, Fr,

Press again to be SA, SU.

Press again to be MO, WE, Fr;

Press again to be TU, TH, SA;

Press again to be MO, TU, WE;

Press again to be TH, Fr, SA;

Press again shall return to the original dates.

Set time: Press hour key once to be one hour, and press minute once to be one minute, the second value to be zero when setting minute value. The mode is 24 times, set by rolling up and down.

E、Connection methods and size

Connection methods and outline size as picture 2.

Enlarge capacity method: can enlarge the capacity by controlling AC contactor.

V、Operation example

For example, one electrical water heater, shall set open and close time as below required:

Open 3 times everyday from Monday to Friday, First time open at 6:15 A.M, and close at 8:00 A.M, second time open at 11:00 A.M, and close at 13:00 P.M, and third time open at 17:30 A.M, and close at 22:00 P.M.  (Note: time setting in order)

(1) Preparation before program setting

Before setting AUTO ON program, you shall check the machine time indicated, if it is not the present Beijing time, you shall adjust as C term of section 4.

（2）Set auto on/off program

At open lock situation: Press set key to enter the first close time setting situation of timing program.

LCD shows “1ON” at the down left corner, which means program No, and the time shows “——:——”, which means close time not set. At this time, press the week key, first set open time, and then press hour and min key to set close time. The methods are subject to d term of section 4. After setting open time, press set key to the first close time setting.

LCD shows “1OFF” at the down left corner, which means program No, and the time shows “——:——”, which means close time not setting. At this time, press the week key, first set open time, and then press hour and min key to set close time. The methods are subject to d term of section 4. After setting open time, press set key to the first close time setting.

LCD shows “2ON”at the down left corner, which means program No, and the time shows “——:——”, which means close time not setting. At this time, press the week key, first set open time, and then press hour and min key to set close time. The methods are subject to d term of section 4. After setting open time, press set key to the first close time setting.

LCD shows“2OFF”at the down left corner, which means program No, and the time shows “——:——”, which means close time not setting. At this time, press the week key, first set open time, and then press hour and min key to set close time. The methods are subject to d term of section 4. After setting open time, you shall go on setting control program same as above, till 8 groups finish. If you don’t need setting 8 groups, you shall press clock key to the clock indicate situation after setting relative open and close time, thus the program operation finishes. You also can operate nothing within 15s, and auto enter the clock situation and auto lock.

VI. Use methods

After setting all ON/OFF program, press mode key, the timer switch change to auto working situation, and auto operate as required after putting through the power source.

If operate at no setting and normal open situation, you can press mode key, LCD shows “ON” at its bottom , at this time the power source indicator open, means output power source has been open. If required timer switch to close at once, you can press mode key, the LCD show “OFF” at its bottom, at this time the power source indicator close, means output power supply has been close.

VII、Notice items

1. When use set key to set open/close program, you must check the input date whether correct or not.

2. Only at the auto mode working situation, the machine shall operate under the set program.

3. The timer switch is at auto operation mode, if press the mode key to change the working mode, the timer switch shall not execute the program, and operate as the old mode, open-auto-close……)

4. IF you haven’t use it for long time, and the inside battery voltage not full and LCD shows nothing, you shall connect the input terminal of timer switch to the power source for charging, then the LCD shall show clock, and the machine shall return all functions, and use normally.

5. IF the machine doesn’t be used normally, special persons shall repair, and dismantling by person is prohibited.

The product is guaranteed maintain for 1 year form purchase date of the user.